So net buildup of holes or electrons in either regionÞ

Jpdrift+Jpdiffusion=0				(1a)(5-2)

Jndrift+Jndiffusion=0				(1b)

· the e field is non-zero only in a region of width

W (Fig 5-9)

· e is related to the electrostatic potential

ex=-dVxdx								(2a)

or

Vx=-−xxexdx								(2b)

since

0, x<-w2

- e(x) = 		-f(x), −w2<x<w2

0,x>w2

we have

Vx=−w2w2fxdx+C

· the region W is called

-the transistion region,

-space charge region, or

-depletion region

· V0 is called

-the contact potential

-diffusion potential, or

-built-in voltage

· the amount of band-bending at equilibrium is qV0 (Fig. 5-7)

· the electrostatic potential is related to electron energy by E=-qV

· Vn > Vp Þ electron energies are lower on the n-side (Fig. 5-7)

Equation for Vo (=f(NA,ND))

Jpdrift+Jpdiff=0						(1a)(5-2a)

or,

qmppxex−qDpdpxdx=0				(5)(5-3)

rearranging (5):

mpDpex=1pdpxdx							(6)(5-4)

using the Einstein Relations and (2a):

−qkTdVxdx=1pdpxdx						(7)(5-5)

Integrating (7) from -w/2 to w/2:

−qkTVpVndV=pppndpp								(8)

−qkTVn−Vp=lnpn−lnpp=lnpnpp		(9)(5-6)

Vn−Vp=V0 ,  so

−qV0kT=lnpnpp=-lnpppn

and	Vo=kTqlnpppn						(10)(5-7)

if the doping on the p-side is NA, the doping on the n-side is ND

Pp=NA		(assume 100% ionization)

we can find pn from

nn=ND ,

pnnn=ni2

pn=ni2ND			(100% ionization)

so	V0=kTqlnNAni2Nd=kTqlnNANDni2		(11)(5-8)

from (10, 5-7)

V0=kTqlnpppn

we have

eqV0/kT=pppn						(12)(5-9)

since	ppnp=ni2=nnpn

Þ			pppn=nnnp						(13)

so		V0=kTqlnnnnp						(14)

eqV0/kT=nnnp					(15)(5-10)

Example 5-1  Consider n-type Ge with 											ND=1016cm−3

· alloy junction with In at 160°C (See App.V)

-calculate Ei−EF in p + n regions

-draw the band diagram

-calculate V0

solution:				From Appendix V,

NA@3´1018cm−3

assume 100% ionization

pp=nieEi−EFpkT

niGe=2.5´1013cm−3				(p.79, Fig 3-17)

Ei−EFp=kTlnppni=0.0259ln3´10182.5´1013º11.69

Ei−EF=0.3029eV

on the n-side

nn=nieEFn−EikT

EFn−Ei=kTlnnnni=0.0259ln10162.5´1013»5.9914

EFn−Ei=0.1552eV

qV0=Ecp−Ecn=0.303+0.155eV=0.458eV

V0=0.458V

calculate V0 from (11) or (5-8):

V0=kTqlnNANDni2

V0=0.0259eVeln3´101810162.5´10132

6.25´1026

V0=0.0259ln3´10346.25´1026=0.025917.686...

V0=0.458V

Equilibrium Fermi Levels

dEFdxº0⇒EFp=EFn

recall that	pppn=nnnp=eqV0kT	(12,14)(5-9,5-10)

nn=Nce−Ecn−EFnkT							(3-15)

np=Nce−Ecp−EFpkT							(3-15)

so	nnnp=Nce−Ecn−EFnkTNce−Ecp−EFpkT=eqV0kT			(16)

e−Ecn−EFnkTeEcp−EFpkT=eqV0kT	  (17)

Ecp−Ecn=qV0kT						(18)

similarly (see text)

Evp−Evn=qV0kT					(19)5-12)

note:  if we apply bias to the junction, we raise or lower the potential barrier

Þ concept of Fermi level is only valid at equilibrium

-no applied field (or voltage)

-no injected current

-no incident light

{at equilibrium, the junction voltage Vj = V0

We will show later that under non-equilibrium conditions

(20)	Vj=V0−VA			(VA>0, forward bias,

VA<0, reverse bias,)

(21)	Fn−Fp=qVA		(VA>0 or VA = VF )

(22)	Fn−Fp=-qVA		(VA<0 or VA = −VR )

Space Charge at a Junction

-very few carriers in the transition region

-some diffusion and some drift carriers (but just passing thru!)

-approximate space charge in the transition region by the uncompensated donor and acceptor ions (Fig. 5-9b)

· this appproximation is known as the “depletion approximation”

· net charge in the device = 0 (at equilibrium)

-depletion approximation Þ nonzero charge 	regions are only in the depletion region of 	width W

Þ the total negative charge has to equal the total positive charge in the depletion region, where

Q−=-qAxp0NA								(23a)

Q+=qAxn0ND									(23b)

where		A is the cross sectional area of the diode

xp0 is the penetration of the depletion 				region into  the p-material

xn0 is the pentration of the depletion 					region into the n-material

(both defined at equilbrium)

by charge neutrality,

Q−=Q+⇒								(24a)

qAxp0NA=qAxn0ND				(24b)(5-13)

and	W=xp0+xn0					(24c)

(note: I was a little sloppy back on p. 457-460 where I used -w/2 and w/2.)

Review:		Maxwell’s Equations are:

Ñ´ {er,t−¶Br,t¶t					(A)

Ñ´ {Hr,tJr,t+¶Dr,t¶t		(B)

Ñ´ {Dr,t=ρr,t					(C)

Ñ⋅Br,t=0							(D)

where	er,tº electric field intensity vector (V/m)

Hr,tº magnetic field intensity vector(A/M)

Dr,tº electric displacement vector(coulombs/m2)

Br,tº magnetic induction vector (webers/m2)

ρr,t= volume density of charge (coulombs/m3)

r= position vector (m)

t  =  time (seconds)

D=Î {e									(a)

B=mH									(b)

If J is an ohmic conduction current:

J=ser,t						(drift) (c)

and if J is due to other moving charges:

J=ρr,tvr,t				(diffusion) (d)

where (in a “simple” media):

Î=ÎrelÎ0		(farads/m)			(i)

m=mrelm0	(henrys/m)			(ii)

and always

vr,t = velocity				(iii)

s = conductivity				(iv)

more detail:

Î0=8.854´10−12		farads/m

m0=1.2566´10−6		henrys/m

Îº electric permittivity

Îrº relative permittivity or dielectric constant

nºÎr= index of refraction

mº magnetic permeability

mr º relative permeability

*disclaimer:  n¹n,	   m¹mobility

The velocity of an electromagnetic wave (“the speed of 	light”) is

vph=1mÎ

in free space

c=1m0Î0

mrel=Îrel=1					(free space)

c=11.2566´10−68.854´10−12=2.9979´10

m/sec

for most optical materials,

mr=1 ,  so

vph=1mrm0ÎrÎ0=cÎr=cn

One final question--where did m/s come from?

m=henrysmÎ=faradsmQºCV⇒farads=coulombsvolts			 EE2350

yºLi  and  V=dydt=Ldidt			EE2352

so	henrys=voltscoulombs/sec2

so		mÎ⇒sec2m2

***********end of tangent****************

Back on p.472 and in Fig. 5-9 (p.470), we have a charge distribution:

and we can use the divergence equation for the electric field:

Ñ⋅D=ρ						(c)		p.472

which reduces to (in 1-D):

Îdedx=ρ

or		dexdx=ρÎ						(25)

to find exxºex :

dedx=qÎp−n+Nd+−Na−				(26)(5-14)

from Fig. 5.9b (depletion approx):

0,	x<-xp0

p−n+Nd+−Na−=		-Na, −xp0<x<0

Nd,  0<x<xn0

(27)							0,	xn0<x

(assuming 100% ionization)

so integrating (26) to find e (and recalling that e is continuous):

· for x<-xp0,  e º 0				(28a)

· (25) Þ

ex=1Îρx

so for  −xp0<x<0 ,

ex=1Î−xp0x−qNadx=−qNaÎx−xp0x

=−qNaÎ−x--xp0

=qNaÎxp0−x						(29b)

for 0<x<xn0

ex=e0−+1Î0xqNddx

by 29b,	e0−=−qNaÎxp0					(30a)

so		ex=−qNaxp0Î+qÎNdx				(29c)

0<x<xn0

at x=xn0 ,

ex=−qNaxp0Î+qNdxn0Îº0

(by (24b), p.472)

so	ex=0 ,				x>xn0				(29d)

summarizing:

ex=0 , 		x<-xp

ex=−qNaÎxp0−x ,			−xp<x<0

ex=−qNaxp0Î+qÎNdx ,		0<x<xn0

ex=0 , 			xn0<x

which is plotted in Fig. 5-9c.

Recall that e is related to the potential gradient V(x) by

ex=-dVxdx

or

Vx=-exdx

which is the curve shown in Fig. 5-7 (b):

V0 = -(Area in Fig. 5-9c)

=-12We0							(31)

by (30a):

e0=-qNaxp0Î

W=xp0+xn0						(i)

and by charge neutrality:

xp0Na=xn0Nd							(ii)

so		V0=12WqNaxp0Î12WqNdxn0Î    (32)(5-19)

We can manipulate (i) and (ii):

W=xp0+xn0⇒xn0=W−xp0

so		xp0Na=W−xp0Nd

xp0Na=WNd−xp0Nd

xp0Na+Nd=WNd

xp0=WNdNa+Nd						(33)

and we can write (32) as:

V0=q2ÎNaNdNa+NdW2					(34)(5-20)

and we solve (34) for W:

W=2ÎV0qNa+NdNaNd12

=2ÎV0q1Nd+1Na12		(25)(5-21)

using (11) from p.461

V0=kTqlnNANDni2

W=2ÎkTq2lnNANDni21Nd+1Na12	(36)(5-22)

since		xp0=WNdNa+Nd						(37a)