ELECTRICAL ENGINEERING DEPARTMENT
SOUTHERN METHODIST UNIVERSITY
FIBER OPTIC TELECOMMUNICATIONS

SMU EE 5303    NTU TM-513-N	                    Midterm #2	April 17, 1996

Name (print):          ________________________

(Please print your name at the bottom of the remaining pages)

Location:	________________________

(NTU, TAGER, Video, Video on Campus, or Campus-live)

Student ID #:        ________________________

Note #1:    each question is worth 5 points.

Note #2:    Some of the multiple choice questions may have more than one correct answer listed.    If so, circle all of the correct responses for the question.

Note #3:    please provide a brief explanation of your answers to multiple-choice type questions.

Note #4:    Several figures are shown on the last pages that could be helpful in solving some of the problems below.

TIME LIMIT:	80 minutes

ALLOWED:    	calculators, one page (8.5” x 11”--can write on both sides) of notes from the first midterm; another page (8.5” x 11”--can write on both sides) of notes for this midtem.

EXAM:	closed book; students are to work by themselves

1.	A SI fiber has core refractive index of n1 = 1.445, and a cladding index of 
n2 = 1.440.    The normalized frequency is 2.2    If the source wavelength is

 = 1.3 µm:

a)    what is the core radius of this fiber?

b)    what is the numerical aperture (NA) of this fiber?

c)    what is the spot-size of this fiber?

d)    how many modes can propagate in this fiber?    Briefly explain your 	answer.

2.	i) Sketch the refractive index profile of    a step-index (SI) fiber:

ii)    Sketch the ray trajectories inside a step-index fiber.    The open box below represents the core of the step-index fiber and the region outside the core represents the cladding region.

iii) Sketch the refractive index profile of    a graded-index (GRIN) fiber:

.

iv)    Sketch the ray trajectories inside a GRIN fiber.    The open box below represents the core of the GRIN fiber and the region outside the core represents the cladding region.

3.	For a symmetric slab waveguide with index n2= 3.55 and n1 = 3.6, how many modes will propagate if the waveguide thickness is 4.0 µm and the free space wavelength of the light propagating in the waveguide is 1.3 µm? Include both TE and TM polarizations.

4.	Consider an asymmetric slab waveguide with n1 = 2.29, n2 = 1.5, and n3 = 1.0.    What range of thicknesses of the central region (the layer with n1 = 2.29) will support exactly 3 TE and 3 TM modes?

5.	Consider either a symmetric slab waveguide or a circular step index fiber.    If the geometry remains constant, but the wavelength decreases, will the effective index increase or decrease?    Briefly explain.

6.	Consider either a symmetric slab waveguide or a circular step index fiber.    If the geometry remains constant, but the wavelength decreases, will the mode spot size (sometimes called the mode-field size) increase or decrease?    Briefly explain.

7.	Consider either a symmetric slab waveguide or a circular step index fiber.    If the wavelength remains constant, but the index difference n = (n1 - n2) decreases, will the number of modes increase or decrease?    Briefly explain.

8.	If the index of refraction of the core of a single-mode fiber is n1 = 1.455 and that of the cladding is 1.450, what is the maximum value of the core radius?

9.	A semiconductor laser emits optical radiation with a free space wavelength of 1.31 µm.    What is the i) frequency, ii) velocity, and iii) wavelength of the optical radiation inside the semiconductor laser?    The effective index of refraction of the semiconductor laser material is 3.43.

10.	An ideal laser diode has a threshold current of 20 mA and a slope efficiency of 0.25 mW/mA.    Sketch the light (optical output power) versus drive current for this device for a current range of 0 to 100 mA.

11.    i) For the ideal laser described in the previous problem, how much optical power is emitted at a drive current of 50 mA?

ii)    what does the acronym “laser” and “LED” stand for?

12.	Which of the following waveguides have (or has) a cutoff wavelength (for a finite, non-zero slab or core thickness) for the lowest-order mode?    Briefly explain your choice(s).

a)    a symmetric slab waveguide

b)    a step-index circular optical fiber waveguide

c)    an asymmetric slab waveguide

13.    Approximately how many modes will an optical fiber with a core diameter of 62.5 µm and a cladding diameter of 125 µm support if the core index is 1.465 and the cladding index is 1.46? Assume that the radiation propagating in the fiber has a free space wavelength of 1.31 µm.

14.	For a single mode fiber, the V parameter is typically chosen to be between 2 and 2.4.

i)    Why is the lower limit typically about 2?

ii)    Why is the upper limit about 2.4?

15.	Consider a step-index single mode fiber communication system operating at a free space wavelength of = 1.6 µm.    Does material dispersion or waveguide dispersion contribute the most to the total dispersion?    Base your explanation on Figs. 3-8 and 5-23.

16.	Which of the following are advantages of LEDs compared to lasers?

i)    high speed modulation

ii) narrowest linewidth

iii) coupling efficiency to fibers

iv) high power

v) reliability

vi) cost

17.	i)    Identify the following TE modes of a symmetric slab waveguide:

ii)    are the distributions shown above field distributions or intensity distributions?    Briefly explain.

18.	You have to use an LED as a source for a fiber system.    You can choose between single-mode SI, single-mode GRIN, multi-mode SI, and multi-mode GRIN fibers.    What is the best choice (assuming that the core diameters of the single-mode fibers are the same and that the core diameters of the multi-mode fibers are the same)?    Briefly explain your choice.

19.	You are using a DFB laser as a source for a fiber system.    You can choose between single-mode SI, single-mode GRIN, multi-mode SI, and multi-mode GRIN fibers.    What is the best choice (assuming that the core diameters of the single-mode fibers are the same and that the core diameters of the multi-mode fibers are the same)?    Briefly explain your choice.

20.	The Corning data sheets (see next pages) on the Dispersion Compensator Module describes a component that is designed to:

a)    reduce dispersion in 1.3 µm fiber systems

b)    allow fiber originally designed to operate at 1.3 µm to be used with minimum dispersion at 1.55 µm

c)    eliminate dispersion in all fibers

CERTIFICATION OF TESTING ENVIRONMENT:
I certify that I have completed this test in the 80 minute time period allotted, and that I have neither given nor received help from another person, nor have I had advanced access to questions or answers.

Signed:________________________________

OFF-CAMPUS STUDENTS ONLY:
I distributed and collected these test materials in an 80 minute interval

	on_______________________________(date).

Signed__________________________________
Site Coordinator must sign