ELECTRICAL ENGINEERING DEPARTMENT
SOUTHERN METHODIST UNIVERSITY
FIBER OPTIC TELECOMMUNICATIONS

SMU EE 5303    NTU TM-513-N	                    Midterm #1	February 23, 1997

Name (print):          ________________________

Location:	________________________

Note #1:    each question is worth 5 points.

Note #2:    Some of the multiple choice questions may have more than one correct answer listed.    If so, circle all of the correct responses for the question.

Note #3:    Please print your name at the bottom of the remaining pages.

Note #4:    Please indicate your location:    NTU, TAGER, Video Distance, Video on Campus, or Campus-live

Note #5:    please provide a brief explanation of your answers to multiple-choice type questions.

TIME:    The allowed time period is 90 minutes.

CONSTRAINTS:    This exam is closed to books and notes with the exception below:

ALLOWED:    1 page of notes (81/2 x 11; can write on both sides); calculator; straight edge; ruler; and compass.

1.	i)    what is the frequency of an optical plane wave that has a free space wavelength of  = 1.2 µm?

ii)    what is the energy of the photons in this optical wave?

iii)    What is the velocity of this plane wave as it moves through free space (air)?

2.    i)    how many photons/sec with a wavelength of 1.2 µm are required to produce 1 W of optical power?

ii)    what is the velocity of this plane wave in a large block of GaAs which has an index of refraction of 3.5 (at a wavelength of  = 1.2 µm).

3.    	When an electromagnetic wave propagates from one medium with an index of refraction of n1 = 3.4 into a second medium with an index of refraction of n2 = 1.4,

a)    only the frequency changes,

b)    only the wavelength changes,

c)    both the frequency and the wavelength change, and/or

d)    the frequency and the wavelength both remain the same.

Briefly explain your answer:

4.	i)    A plane wave with a wavelength of  = 1.6 µm travels through a block of glass with a velocity determined by:    a) the group velocity, b) the phase velocity, or c) the speed of light in vacuum

ii) A gaussian shaped pulse of light with a bandwidth of 1 GHz centered at a wavelength of  = 1.6 µm travels through a block of glass with a velocity determined by:    a) the group velocity, b) the phase velocity, or c) the speed of light in vacuum

5	i)    what wavelength spread does a 1 GHz frequency spread correspond to at a wavelength of  = 1.6 µm?

ii)    write down the equations for a) the group velocity, b) the phase velocity, and write down the numeric value for the speed of light in vacuum:

6.	i)    A fiber has an optical loss of 0.1 dB/km.    What is the optical loss of a 500 km length of this fiber in units of dBs?

ii)    If 10 mW of optical power coupled into this fiber, what will the output power be after 500 km?

7.	i)    the figure below shows rays incident on the left-hand-side of the lens.    Show the continuation of the rays on the right-hand-side of the lens.

ii)    indicate the image of the object in the figure above.    Roughly, what is the magnification?

8.      A lens system similar to that shown in problem 7 is used to collect the light from a laser diode source (which has an angular spread of 35°) into an optical fiber with an acceptance angle of 10 °.

i)    what is the resulting magnification of the lens system?

ii)    if the diameter of the optical spot at the laser facet is 2 µm, and the core of the fiber is 10 µm, will the image of the optical spot at the facet be larger than, equal to, or less than the core of the fiber?

9.	i)    a light ray proceeds from InGaAsP (n1 = 3.4) into air (n2 = 1.0).    If the incident angle is i = 7°, what is the transmission angle t ?

ii)    now consider a light ray going from air into InGaAsP with the incident angle.    What is the resulting transmission angle t ?

10.	The critical angle can be defined as the incidence angle for which Snell’s Law gives 90° as the transmission angle.    For incidence angles larger than the critical angle, the transmitted angle is complex (or your calculator may say:    “error”).

i)    what is the critical angle for a ray in glass (with an index of 1.45) if the material outside the glass is air (with an index of 1.0)?

ii)    for a critical angle to exist between two media with index n1  and n2,

a)    the light ray must be traveling from a low index to a high index,

b)    the index difference must greater than tan-1(n2/(n1 + n2)),

c)    the loss in the high index medium cannot exceed the loss in the low index 	medium, and/or

d)    the light ray must be traveling from a high index to a low index.

Briefly explain your answer:

11.	A semiconductor laser emits optical radiation with a free space wavelength of 1.31 µm.    What is the i) frequency, ii) velocity, and iii) wavelength of the optical radiation inside the semiconductor laser?    Assume that the index of refraction of the semiconductor laser material is 3.35.

12. i)    To save bandwidth, a new telephone company decides to allocate only 3 kHz to the human voice and to use 7 bits to digitally encode the signal.    What    bandwidth will each voice channel require?

ii)    What is the maximum number of phone calls (voice channels) that an optical fiber communications link can carry at a data rate of 5 Gbps (5 x 109 bps)?    Assume that each voice channel requires the standard bandwidth of 64 kbps.

13.    The near-field distribution at the facet of a semiconductor laser is modelled as a Gaussian distribution with wo = 0.75 µm perpendicular to the junction and wo = 3 µm parallel to the junction.    The free space wavelength of the semiconductor laser is 1.55 µm.    Estimate the far-field beam divergence    i) perpendicular and ii) parallel to the junction of the laser.

14.    A Gaussian light beam is focused to a spot by a lens.    The diameter (2w) of the input beam is 3 mm.    What is the diameter of the spot size (2wo) of the focused spot if the wavelength of the beam is 0.6328 µm and the focal length of the lens is 10 mm?

15.    The principle of least time can be used to derive

a)    Snell's law,

b)    the law of refraction,

c)    the law of reflection,

d)    the thin lens equation,

e)    the graded index profile of a GRIN lens, and/or

e)    the best path for a lifeguard to rescue a drowning person

Briefly explain your answer.

16.    How far does a plane wave with a free space wavelength of 980 nm travel in one femptosecond (10-15 sec) in a material with an index of refraction of 1.45?

17.	The veloctiy that describes the propagation of a pulse or an optical signal that contains information is

a)    the group velocity

b)    the phase velocity

and the velocity that describes the propagation of a plane wave or a ray is

c)    the group velocity

d)    the phase velocity

Briefly explain your answers.

18.    Explain why the reflected angle is equal to the incidence angle when a ray is incident on a planar boundary between two materials with different indices of refraction.    (Sketches and words are a good approach.)

19.    A laser pointer with a spot size (2wo) of 2 mm is aimed at a reflective road sign that is 10 meters away.    Assume that the light distribution from the laser pointer is Gaussian.    The wavelength of the laser pointer is 680 nm.    What is the spot size (2w) on the road sign?

20.    A lens with a focal length of 1 cm is used to focus light with a wavelength of 630 nm to a spot.    In one case, light of a uniform intensity (across the lens) is focussed.    In the second case, light with a Gaussian distribution (across the lens) is focussed.

i)    describe the distribution of the light at the focus in both cases.

ii)    which intensity distribution at the lens results in the smallest spot size at the focus?

CERTIFICATION OF TESTING ENVIRONMENT:
I certify that I have completed this test in the 90 minute time period allotted, and that I have neither given nor received help from another person, nor have I had advanced access to questions or answers.

Signed:________________________date:________

OFF-CAMPUS STUDENTS ONLY:
I distributed and collected these test materials in an 90 minute interval

	on_______________________________(date).

Signed__________________________________
Site Coordinator must sign